For Windows

1.Copy the windows package on the desktop of MINI PC

2.Select "FlashBios.bat" file and run it with administrator. Update Process will start automatically.

3.Restart the PC after the process finish,check the bios version

For EFI

- Copy all the file under "EFI" folder to the root direct of USB driver,which format is FAT32

2.Power on MINI PC and keep clicking "delete" key untill into bios settings interface.

3.Disable "Security Boot" By the path: Security->Security Boot->Disable

4.Input the USB driver to USB port of MINI PC

5.Restart the PC and keep click F11 until goto boot option, select the usb driver option as boot option

6.Update Process will start automatically.

7.Restart the PC after the process finish,check the bios version